VersaIntegrator Installation and Configuration Process

Meadowbrook Terrace

- Download the install package zip file from http://collopps.com/VersaIntegrator/ and save in root of C:\

- Unzip all install files

- Create C:\Logfiles directory

- Plug in R280 USB Dongle to computer and allow windows to recognize and install drivers

- Go to install files folder and launch Zadig

  - In Zadig, go to Options -> List All Devices to see a listing of USB devices

  - Select Bulk In, Interface 0 and replace the driver with the WinUSB driver ending in 16385

  - When driver installed, quit Zadig

- Go to install files folder and install VB Cable (both versions).  Make sure to Run as Administrator when launching the .exe files

- Go to install files folder and install sdrsharp

  - Go to sdrsharp-x86 folder

  - Run the install-rtlsdr.bat file

  - When batch process is complete, verify sdriq.dll and rtlsdr.dll files are in the folder

  - Copy SDRShart.AccessibilityPlugin.dll into sdrsharp-x86 folder

  - Open Plugins.xml file and add the following line to it:

<add key="Accessibility Control" value="SDRSharp.AccessibilityPlugin.AccessibilityPlugin,SDRSharp.AccessibilityPlugin"/>

  - Save the plugins.xml file

- Go to control panel, sound settings

  - Click on recording tag

  - Set CABLE Output as the default recording device if not already selected

- Launch SDRSharp application

  - Select Source RTL-SDR (USB)

  - On Radio tab configure the following:

    - Select NFM

    - Filter: Blackman Harris 4

    - Bandwidth: 32,000

    - Order:  10

  - On Audio tab configure the following:

    - Output:  Windows Direct Source CABLE

    - Uncheck Filter Audio

  - On Frequency Manager configure the following:

    - Create a new entry

    - Set to POCSAG frequency of Visiplex or other paging unit (462,750,000 for Meadowbrook)

    - Save (creates frequencies.xml file with these settings)

  - Click on Setting Gear and configure the following:

    - Sample Rate: 1.4 msps

    - RF Gain:  42.1dB

  - Ensure the volume is set to 50db

- Launch PDW

  - Click on Options in menu bar

  - In POCSAG Section configure the following

    - Enable POCSAG Decoding

    - 512

    - 1200

    - 2400

    - If no accurate…

  - Click on Interface -> Setup (Ctrl-S)

    - Select checkbox for Soundcard

    - Verify CABLE Output selected

  - Click Options –> General (Ctrl-G)

    - Next to “Path for Logfiles” field, click on the Browse button.  Select the C:\Logfiles folder created in Step 3 above.

  - Click Monitor from menu and ensure POCSAG/FLEX is checked

- Download and install Macro Express Pro  from https://www.macros.com/

  - Create Macro (modify sample one or create new)

  - Macro should contain the exact steps configured below

A copy of the Macro above can be found here:  https://drive.google.com/drive/folders/1hwfO5nDtFt64z_SaWDH8dbfQOBaQc7NB?usp=sharing

- Setup PDW to run automatically on boot

  - Open windows explorer

  - Type in %APPDATA%\Microsoft\Windows\Start Menu\Programs\Startup

  - Open new windows explorer tab

  - Go to PDW folder

  - Copy the exe

  - Go to startup folder

  - Paste as shortcut in the folder

- Install VB Integrator

  - API URL:  http://clientnameadmin.versabadge.com/api/v3

  - Org ID – client assigned Org ID in DB

  - Log File Path:  C:\Logfiles\

Meadowbrook Example:

API URL:  http://meadowbrookadmin.versabadge.com/api/v3

Org ID:  3

Log Path:  C:\Logfiles\

- Reboot computer

- Verify SDR# and PDW run automatically after boot is completed

- Test sending signal from pager and see if it is seen in SDRSharp and decoded in PDW as well as received on designated cell phones

Other Configuration Steps

- Configure Display / Sleep settings

  - Never sleep on battery

  - Never sleep when plugged in

- Configure Computer reboot schedule

  - https://answers.microsoft.com/en-us/windows/forum/windows_7-performance/how-can-i-set-up-a-task-in-task-scheduler-to/b491983a-8aa2-4603-ad86-375ce2647148?auth=1

- Install TeamViewer and test connectivity

  - Email:  teamviewer@versabadge.com

  - Password:  Typical PNX admin password for VB