- Copy all install files and folder to c:/

- Unzip all install files

- Plug in R280 USB Dongle to computer and allow windows to recognize and install drivers

- Go to install files folder and launch Zadig

  - In Zadig, go to Options -> List All Devices to see a listing of USB devices

  - Select Bulk In, Interface 0 and replace the driver with the WinUSB driver ending in 16385

  - When driver installed, quit Zadig

- Go to install files folder and install VB Cable (both versions).  Make sure to Run as Administrator when launching the .exe files

- Go to install files folder and install sdrsharp

  - Go to sdrsharp-x86 folder

  - Run the install-rtlsdr.bat file

  - When batch process is complete, verify sdriq.dll and rtlsdr.dll files are in the folder

- Go to control panel, sound settings

  - Click on recording tag

  - Set CABLE Output as the default recording device if not already selected

- Launch SDRSharp application

  - Select Source RTL-SDR (USB)

  - On Radio tab configure the following:

    - Select NFM

    - Filter: Blackman Harris 4

    - Bandwidth: 32,000

    - Order:  10

  - On Audio tab configure the following:

    - Output:  Windows Direct Source CABLE

    - Uncheck Filter Audio

  - On Frequency Manager configure the following:

    - Create a new entry

    - Set to POCSAG frequency of Visiplex or other paging unit

    - Save

  - Click on Setting Gear and configure the following:

    - Sample Rate: 1.4 msps

    - RF Gain:  42.1dB

- Launch PDW

  - Click on Options in menu bar

  - In POCSAG Section configure the following

    - Enable POCSAG Deconding

    - 512

    - 1200

    - 2400

    - If no accurate…

- Test sending signal from pager and see if it is seen in SDRSharp and decoded in PDW